Viscoelastic Film in a liquid.

The current simulation runs were to create a layer thick enough in comparison with the bulk and has different viscous properties as well.

The runs were fine-tuned to achieve results, which were closer to the analytical calculated values.

The following table lists the achieved values in comparison with the analytical results:

| Radius of the Sphere | Grid resolution | Simulated Result df (Hz) | Analytical Result df (Hz) | Error df (%) | Simulated Result dG (Hz) | Analytical Result dG (Hz) | Error dg (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 nm | 0.5 nm | -771.36 | -771.36 | 0 | 718.99 | 718.99 | 0 |
| 20 nm | 0.33 nm | -771.36 | -771.36 | 0 | 718.99 | 718.99 | 0 |
| 40 nm | 0.5 nm | -821.40 | -821.40 | 0 | 725.30 | 725.29 | 0.001 |
| 40 nm | 0.33 nm | -821.40 | -821.40 | 0 | 725.29 | 725.29 | 0 |
| 60 nm | 0.5 nm | -867.93 | -866.86 | 0.12 | 735.57 | 735.27 | 0.04 |
| 60 nm | 0.33 nm | -866.86 | -866.86 | 0 | 735.27 | 735.27 | 0 |
| 80 nm | 0.5 nm | -907.69 | -907.69 | 0 | 748.44 | 748.44 | 0 |
| 80 nm | 0.33 nm | -907.69 | -907.69 | 0 | 748.44 | 748.44 | 0 |

The following were the used parameters and are constant throughout the simulation runs.

- Number of cycles = 100

- Target accuracy in Hz = 0.001

- NX = NZ = 1

- Cell diameter / sphere diameter = 2

- Truncation of sphere = 0.5 (50%)

- iovt (number of overtones) = 1

- nufac. (Viscosity of sphere in comparison to bulk) = 2  (2 *bulk medium)

Approach:

At the beginning, we tried to vary cell height and grid resolution at the same time and we could clearly observe that the results had a significant effect. There was no uniform change and the values seemed to fluctuate.

Then, after a discussion, the grid resolutions was varied without changing the cell height. However, added to this was the change in the radius of the sphere in the bulk, changed in intervals for every sphere radius.

The radii of sphere was increased in steps of 20 nm and tested, to observe if we could reach our analytical solution without increased or no errors.

Using the value of tandelinv = 0, meant that

tanδ=G''G'= η'η''

tandelinv=1tanδ=η''η'

Where η’’ is storage viscosity and η’ is the dynamic viscosity.

Below is a screenshot of the result for the increasing radii of sphere and the values of df and dG. We clearly observe the much-desired increasing dG values. (The cell height here was 80 nm)